Lab 02

PC Network TCP/IP Configuration

OBJECTIVES OF THE LAB

Following topics will be covered in this lab

- Gather information including connection, host name, Layer 2 MAC address and Layer 3   TCP/IP network address information.

- Compare network information to other PCs on the network.

- Identify tool used for discovering a computer’s network configuration.

ABOUT IPCONFIG

ipconfig (Internet Protocol Configuration) in Microsoft Windows is a console application. It can be used from MS-DOS shell to display the network settings currently assigned and given by a network. This command can be utilized to verify a network connection as well as to verify network settings. Usage

ipconfig [/allcompartments] [ /? | /all | /renew [adapter] | release [adapter] |/flushdns | /displaydns /registerdns | /showclassid adapter | /setclassid adapter [classidtoset] ] Option Description

================================================

| /? |  | Display help message |
| --- | --- | --- |
| /all |  | Display full configuration information |
| /allcompartments |  | Display information for all compartments |
| /release |  | Release the IP address for the specified adapter |
| /renew |  | Renew the IP address for the specified adapter |
| /flushdns |  | Removes the DNS Resolver cache |
| /registerdns |  | Refreshes all DHCP leases and re-registers DNS name |
| /displaydns |  | Display the contents of the DNS Resolver Cache |

/showclassid  	Displays all the DHCP class IDs allowed for adapter

/setclassid  	 	 	Modifies the DHCP class ID

================================================

The default is to display only the IP address, subnet mask and default gateway for each adapter bound to TCP/IP.

TASKS

Gathering TCP/IP configuration information

Step 1

Establish and verify connectivity to the Internet. This ensures the computer has an IP address.

## Step 2

Use the Start menu to open the Command Prompt, an MS-DOS-like window. Press Start >

Programs > Accessories > Command Prompt

OR

Start > Programs > Command Prompt.

OR

Press Start>Run Then type cmd.

The following figure shows the Command screen. Type ipconfig and press the Enter key. The spelling of ipconfig is critical while case is not. It is short for IP Configuration.

## Figure 1.1 Command Screen for ipconfig

## Figure 1.2 Command Screen for ipconfig

This first screen shows the IP address, subnet mask, and default gateway. The IP address and the default gateway should be in the same network or subnet, otherwise this host would not be able to communicate outside the network. In the figure the subnet mask tells us that the first three octets must be the same to be in the same network.

Record the following information for this computer.

## Step 3

Record the following TCP/IP information for atleast THREE computers

|  | Computer 1 | Computer 2 (Neighbor 1) | Computer 3 (Neighbor 1) |
| --- | --- | --- | --- |
| IP Address | 192.168.137.210 | 192.168.137.169 | 192.168.137.153 |
| Subnet Mask | 255.255.255.0 | 255.255.255.0 | 255.255.255.0 |
| Default Gateway | 192.168.137.1 | 192.168.137.1 | 192.168.137.1 |
| DNS Address | 192.168.137.1 | 192.168.137.1 | 192.168.137.1 |
| DHCP Address | 192.168.137.1 | 192.168.137.1 | 192.168.137.1 |

## Step 4

Compare the TCP/IP configuration of this computer to others on the LAN

If this computer is on a LAN, compare the information of several machines.

Are there any similarities?  Since the first 3 octets are same because they are connected to the same network.

What is similar about the IP addresses? First three octets are similar.

What is similar about the default gateways? All the request go from the same gateway from all the computers.

The IP addresses should share the same network portion. All machines in the LAN should share the same default gateway.

Step 5

Check additional TCP/IP configuration information

To see detailed information, type ipconfig /all and press Enter. The figure shows the detailed IP configuration screen.

## Figure1.3 Command Screen for ipconfig /all

The host name, including the computer name and NetBIOS name should be displayed. Also, the  DHCP server address, if used, and the date the IP lease starts and ends should be displayed. Look over the information. Entries for the DNS, used in name resolution servers, may also be present.  The previous figure reveals that the router is performing both DHCP and DNS services for this network. This would likely be a small office or home office (SOHO) or small branch office implementation.

Notice the Physical Address (MAC) and the NIC model (Description).

MAC: 74-E5-F9-77-C4-7D

NIC: Intel(R) Dual Band Wireless-AC 3165

Write down the IP addresses of any servers listed:

192.168.137.1

Write down the computer Host Name:

AKRCY

Write down the Host Names of a couple other computers:

DESKTOP-UUDNBKL
ZVBI

## Step 6

Close the screen when finished examining network settings.

Repeat the previous steps as necessary. Make sure that it is possible to return to and interpret this screen.

Based on observations, what can be deduced about the following results taken from three computers connected to one switch?

Computer 1

IP Address: 192.168.12.13

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.12.1

Computer 2

IP Address: 192.168.12.5

# Subnet Mask: 255.255.255.0

# Default Gateway: 192.168.12.1

Computer 3

IP Address: 192.168.13.97

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.13.1

Should they be able to talk to each other?

Computer 1 and Computer 2 can talk to each other but Computer 3 cannot talk to any of them

Are they all on the same network? Why or why not?

Computer 1 and Computer 2 are on the same network because they have same gateway but Computer 3 have different gateway

Investigate the network using “ping”, “tracert”

# ABOUT PING

The original PING command stood for "Packet Internet Groper", and was a package of diagnostic utilities used by DARPA personnel to test the performance of the ARPANET. However, the modern Internet Ping command refers to a program written by Mike Muss in December, 1983, which has since become one of the most versatile and widely used diagnostic tools on the Internet.

Typical Diagnostic Tests Performed By Ping Command

Some of the internet diagnostic tests performed by ping command are:

- Access – Ping is used to determine whether the remote host is active or inactive. If a certain site is not pinged, but the other sites can, then it's a pretty good sign that your Internet network is fine and that site is down. On the other hand, if you can't ping any site, then likely your entire network connection is down that needs rebooting.

- Time & distance – Another use of Ping command is to determine how long it takes to bounce a packet off of another site. Thereby giving Internet distance in network terms. For example, a web site hosted on your neighbor's computer with a different Internet service provider (ISP) might go through more routers and be farther away in network distance than a site on the other side of the ocean with a direct connection to the Internet backbone. If a site seems slow, then ping distance of that site can be compared with that of other Internet sites to find out whether it is the site, the network, or your system that is slow. You can also compare ping times to get an idea of which sites have the fastest network access and would be most efficient for downloading, chatting, and other applications.

- Domain IP address – Typically, Ping command is used to probe either a domain name or an IP address; if a domain name is pinged, and then it displays the corresponding IP address in its response.

## Usage

ping [-t] [-a] [-n count] [-l size] [-f] [-i TTL] [-v TOS] [-r count] [-s count] [[-j host-list] | [-k host-list]] [-w timeout] target_name where target_name can be either IP Address or host name.

## Option Description

===================================================

-t Ping the specified host until stopped. To see stats &

Continue, type Control-Break; to stop, type

Control-C

-a (Resolve Addresses) Resolve address to host names

-n count (Echo Count) Sends echo packets specified by count; Default is 4

-l size (Send buffer Size) Use to increase or decrease the size of the ICMP packets sent in the ping request;

Min=0, Max=65500

-i TTL (Time to Live) set the number of hops (routers) that the ping request can traverse before it is discarded. The TTL field in the packet is decremented by one each time it passes through a router. When the number reaches zero, the router discards the packet and sends a TTL Expired ICMP message back.

-w timeout Timeout in seconds that ping waits for each reply

==================================================

# EXAMPLES

## Ping a Local host

In this example, a local host is pinged to see if the computer is able to send and receive information.

## Step 1

Use the Start menu to open the command-prompt:

Start>Programs>Accessories>Command Prompt or

Start>Programs>Command Prompt

## Step 2

Type ping, space, and 192.168.2.x. Press Enter key.

Since Ping uses the Internet Control Message Protocol (ICMP) echo-request and echo-reply feature to test physical connectivity, it reports on four attempts and gives an indication the reliability of the connection. Look over the result and verify that the ping was successful. Was the ping successful? A successful host ping is shown in Figure 2.1.

Figure 2.1: Command Screen for ping www.yahoo.com The first output line shows the Fully Qualified Domain Name (FQDN) followed by the IP address.

A Domain Name Service (DNS) server somewhere in the network was able to resolve the name to an IP address. DNS servers resolve domain names, not hostnames, to IP addresses.

Without this name resolution, the ping would have failed because TCP/IP only understands valid IP addresses. It would not be possible to use the web browser without this name resolution.

With DNS, connectivity to computers on the Internet can be verified using a familiar web address, or domain name, without having to know the actual IP address. If the nearest DNS server does not know the IP address, the server asks a DNS server higher in the Internet structure.

## --------------------------------------------TASK --------------------------------------------

- Ping the IP address of the Default Gateway and DNS Servers. Was the result successful?

- Ping the computer’s loop-back address. Type the following command: >> ping 127.0.0.1

c)What is the IP Address of www.yahoo.com: 142.250.194.46

How much time did our ping took to reach www.yahoo.com:2637ms

- Ping the hostname of another computer. Try to ping the hostname of the computer that was recorded in the previous task.

- Ping the hostname of another computer using –t. Try to ping repetitively, the hostname of the computer

- How can we stop the ping?

By pressing CTRL+C

- ping the IP address of a DHCP or DNS server

## Use of Ping Option: Resolve Address

In this example, a website is pinged using its IP Address. By means of ping option: Resolve Address (-a), this IP Address is resolved into its respective host name. Also the ping option:

Echo Count (-n count) is used to send only 2 echo packets. The details are shown in Figure 2.2.

Figure 2.2 Use of Address Resolution Ping Option

## ABOUT TRACERT

Tracert is TCP/IP abbreviation for trace route. The preceding figure shows the successful result when running tracert from Bavaria in Germany. The first output line shows the FQDN followed by the IP address. Therefore, a DNS server was able to resolve the name to an IP address. Then there are listings of all routers the tracert requests had to pass through to get to the destination. tracert uses the same echo requests and replies as the ping command but in a slightly different way. Observe that tracert actually contacted each router three times. Compare the results to determine the consistency of the route. Notice in the above example that there were relatively long delays after router 11 and 13, possibly due to congestion. The main thing is that there seems to be relatively consistent connectivity. Each router represents a point where one network connects to another network and the packet was forwarded through.

## Usage

The tracert command is used to visually see a network packet being sent and received and the amount of hops required for that packet to get to its destination. Trace route (tracert) is a command-line utility used to trace the path that an Internet Protocol (IP) packet takes to its destination from a source. Tracert will determine the path taken to a destination. It does this by sending Internet Control Message Protocol (ICMP) Echo Request messages to the destination. When sending traffic to the destination, it will increment the Time to Live (TTL) field values to aid in finding the path taken to that destination address. The path is outlined from this process.

tracert [-d] [-h maxaimum_hops] [-j host-list] [-w timeout] target_name where target_name can be either IP Address or host name.

## Option Description

===================================================

-d (Do Not Resolve Addresses) Displays the route using numeric addresses only rather than showing both IP address and host names, for faster display.

-h maximum_hops (Max.  Hops) Specifies the maximum number of hops to use for tracing; Default is 30

-w timeout Specifies how long to wait for a reply to each

Request in seconds;

===================================================

## Step 1

Trace the route to the Yahoo web site

Type tracert www.yaho.com and press Enter.

The result shows the complete route to the site, along with the number of hops in the path.

Figure 2.3 Command Screen for tracert www.yahoo.com

--------------------------------------------TASK 02  (3 marks, 2 marks for practicing other options for tracert and ping commands, and Clear documentation)-------------------------------

-a) Trace the route to the GOOGLE PAKISTAN website by typing:

## >> tracert www.google.com.pk

- Trace the route to the  FAST website using options listed in option description table.

>> tracert -d www.nu.edu.pk

- What is the difference between the following commands?

Tracert www.yahoo.com

Tracert –h 20 www.yahoo.com

In the first command it will use 30 hops in 2nd command it is using only 20 hops